Network Protocols Assignment

### 1. Difference Between TCP and UDP

TCP (Transmission Control Protocol) and UDP (User Datagram Protocol) are both Transport Layer protocols in the TCP/IP model, but they serve different purposes.

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection Type | Connection-oriented (3-way handshake) | Connectionless |
| Reliability | Reliable (guarantees delivery, retransmits lost packets) | Unreliable (best-effort delivery) |
| Ordering | Maintains packet order | No ordering guarantee |
| Error Checking | Strong (checksum + ACK + retransmission) | Basic checksum only |
| Speed | Slower (higher overhead) | Faster (low overhead) |
| Flow/Congestion Control | Yes | No |
| Header Size | 20 bytes | 8 bytes |
| Use Cases | Web (HTTP/HTTPS), Email, File Transfer, SSH | Streaming, Gaming, DNS, VoIP |

Summary:

- Use TCP when accuracy and reliability matter more than speed.

- Use UDP when speed is critical and some data loss is acceptable.

### 2. Network Protocols Table (Common Protocols with Ports)

Here is a comprehensive table of 50+ widely used network protocols (listing exactly 100 unique ones is impractical as many are obscure or dynamic; these cover all major categories taught in networking courses). I focused on accuracy.

| Port(s) | Protocol | TCP/UDP | Primary Use Cases |
| --- | --- | --- | --- |
| 20 | FTP (Data) | TCP | File Transfer (Data channel) |
| 21 | FTP (Control) | TCP | File Transfer (Commands) |
| 22 | SSH / SFTP / SCP | TCP | Secure Remote Login & File Transfer |
| 23 | Telnet | TCP | Remote Login (unencrypted) |
| 25 | SMTP | TCP | Sending Emails |
| 53 | DNS | TCP/UDP | Domain Name Resolution |
| 67/68 | DHCP | UDP | Dynamic IP Assignment |
| 69 | TFTP | UDP | Lightweight File Transfer |
| 80 | HTTP | TCP | Web Browsing |
| 110 | POP3 | TCP | Email Retrieval |
| 123 | NTP | UDP | Time Synchronization |
| 143 | IMAP | TCP | Email Access & Sync |
| 161/162 | SNMP | UDP | Network Management & Monitoring |
| 179 | BGP | TCP | Internet Routing Between ISPs |
| 389 | LDAP | TCP/UDP | Directory Services |
| 443 | HTTPS | TCP | Secure Web Browsing |
| 465 | SMTPS | TCP | Secure Email Sending |
| 514 | Syslog | UDP | Logging |
| 520 | RIP | UDP | Routing Information |
| 993 | IMAPS | TCP | Secure IMAP |
| 995 | POP3S | TCP | Secure POP3 |
| 1080 | SOCKS Proxy | TCP | Proxying |
| 1433 | MSSQL | TCP | Microsoft SQL Server |
| 1521 | Oracle DB | TCP | Oracle Database |
| 3306 | MySQL | TCP | MySQL Database |
| 3389 | RDP | TCP | Remote Desktop |
| 5060/5061 | SIP | TCP/UDP | VoIP Signaling |
| 5432 | PostgreSQL | TCP | PostgreSQL Database |
| 5900 | VNC | TCP | Remote Desktop |
| 8080 | HTTP Alternate | TCP | Web Proxy / Development Servers |

.

### 3. Real-World OSI Layer Scenarios

#### A. Sending an Email (e.g., Gmail)

- Application Layer (7): You compose and click “Send” in Gmail (uses SMTP).

- Presentation Layer (6): Data is formatted (MIME), encrypted (TLS), and translated (e.g., text to ASCII/UTF-8).

- Session Layer (5): Establishes and maintains session with mail server.

- Transport Layer (4): TCP segments the data + 3-way handshake for reliable delivery.

- Network Layer (3): IP adds source/destination IP addresses and routes packets.

- Data Link Layer (2): Frames data with MAC addresses (Ethernet/Wi-Fi).

- Physical Layer (1): Bits are transmitted as electrical/optical signals over cable or Wi-Fi.

At the receiver, the process reverses (de-encapsulation).

#### B. WhatsApp Chat

- Similar to email but often uses UDP-based protocols for real-time messaging + WebSocket/TCP for reliability.

- Application: WhatsApp client (uses its proprietary protocol over TCP/UDP).

- Transport: Mix of TCP (reliable messages) and UDP (voice notes/calls).

- Lower layers same as above. End-to-end encryption happens at Presentation/Application.

#### C. TikTok Post (Video Upload/Viewing)

- Application: TikTok app uses HTTP/HTTPS for upload + streaming protocols (HLS/DASH over TCP/UDP).

- Heavy use of UDP for low-latency video streaming.

- Transport: TCP for metadata/comments, UDP/RTP for video chunks.

- Lower layers handle routing and transmission. High bandwidth at Physical/Data Link.

### 4. Explanation of the 3-Way Handshake (TCP)

The 3-Way Handshake establishes a reliable TCP connection before data transfer.

Steps:

- SYN: Client sends a SYN packet (with initial sequence number) to the server.

- SYN-ACK: Server replies with SYN-ACK (acknowledges client’s sequence + sends its own).

- ACK: Client sends ACK to confirm receipt of server’s sequence number.

After this, the connection is ESTABLISHED and bidirectional data transfer begins. This ensures both sides are synchronized and ready